EXPERIMENT 3

ENERGY AND POWER ANALYSIS OF SIGNALS DATE: 14.06.26

OBJECTIVE:

(i)	To calculate the energy and average power of given continuous-time and discrete time signals and classify them as energy signals, power signals, or neither based on their energy and power

characteristics.

PROCEDURE

- Define the time vector and generate the selected signals.

- Compute the squared magnitude of each signal.

- Numerically integrate over a sufficiently large interval for energy.

- Compute average power over an interval containing several periods for the periodic signal.

- Compare numerical results with analytical expectations.

- Classify the signals and record observations.

CODE

clc; clear; close all; dt = 0.001;

t = 0:dt:5;

xE = exp(-t);

xP = sin(2*pi*5*t);

E_num = trapz(t,abs(xE).^2); T0 = 1/5;

idx = t <= 2;

P_num = (1/(t(find(idx,1,'last'))-t(1))) *trapz(t(idx),abs(xP(idx)).^2); figure;

subplot(2,1,1); plot(t,xE); grid on;

title('Energy Signal e^{-t}u(t)');

xlabel('Time (s)'); ylabel('Amplitude'); subplot(2,1,2);

plot(t,xP); grid on;

title('Power Signal sin(2pi5t)'); xlabel('Time (s)'); ylabel('Amplitude'); fprintf('Numerical energy = %g\n',E_num);

fprintf('Numerical average power = %g\n',P_num);

OUTPUT

RESULT

Numerical energy = 0.499977 Numerical average power = 0.5

Energy and average power were calculated successfully, and the given signals were classified as energy and power signals based on their energy and power characteristics.

LABORATORY PERFORMANCE ASSESSMENT RUBRICS:

| Objective | Assessment Criterion | Excellent – 4 | Good – 3 | Satisfactory – 2 | Needs Improve ment – 1 | Weight age | Marks given | Marks obtai ned |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calculate the energy and average power of given signals | Energy & Power Calculation | Correctly calculates signal energy and average power using appropriate mathematical expressions and MATLAB implementation independently. | Performs calculation s correctly with minor numerical or coding errors. | Performs basic calculations but requires guidance or has noticeable errors. | Unable to correctly calculate energy or power. | 25 | 2.5 |  |
| Distinguish energy signals and power signals | Signal Classificatio n | Correctly classifies signals as energy or power signals based on calculated values and theoretical conditions. | Correctly classifies signals with minor errors or limited justificatio n. | Performs basic classification but requires assistance or provides incomplete justification. | Unable to classify the signal correctly. | 25 | 2.5 |  |
| Implement numerical energy/power computation using MATLAB | MATLAB Implementat ion & Numerical Analysis | Develops accurate MATLAB code, obtains reliable numerical values and appropriately selects the integration/summation interval. | Code and numerical results are mostly correct with minor errors. | Implements the basic computation with assistance or noticeable numerical errors. | Unable to impleme nt the required MATLA B computat ion. | 25 | 2.5 |  |
| Compare analytical and simulated results | Theoretical Validation & Comparison | Accurately compares theoretical and simulated values, calculates deviation where applicable and explains discrepancies clearly. | Provides correct compariso n with minor deviations and | Provides basic comparison but validation or interpretatio n is limited. | Unable to compare or validate the results. | 25 | 2.5 |  |